TRƯỜNG ĐẠI HỌC CÔNG NGHỆ KỸ THUẬT TP. HCM

MACHINE VISION PROJECT REPORT

TOPIC: Badminton Player Activity Heatmap

GROUP MEMBER NAMES AND ID

PHẠM VĂN THỊNH - 22146056

TRẦN ANH KHƯƠNG - 22146025

MACHINE VISION PROJECT REPORT

Topic: Badminton Player Activity Heatmap

I. Introductions:

1. Background:

In competitive badminton, understanding player movement patterns is crucial for tactical analysis, performance optimization, and opponent scouting. Coaches and analysts need to identify which areas of the court a player dominates, their movement efficiency, and positional tendencies during rallies.

2. Problem statement:

Video footage from standard cameras captures matches from an oblique (side or elevated) angle, creating perspective distortion. This distortion means that:

-Players near the camera appear larger and occupy more pixels.

-Players far from the camera appear smaller.

-Direct pixel coordinates cannot accurately represent real court positions.

Additionally, No advanced ML models are used like Yolo or Mediapipe.

3. Objectives:

- -Manually calibrate the court using homography (by clicking 4 corners).

- -Detect players using traditional computer vision (background subtraction/motion detection).

- -Extract foot positions (lowest point of player blob).

- -Transform pixel coordinates to real-world court space.

-Generate heatmap over 10 minutes of match segments.

4. Scope and Limitations:

- Input: Static camera video (10 minutes, 30 fps, caulong1.mp4).

- Output: JET-colormap heatmap overlaid on a standard badminton court diagram.

- Assumptions:

- -Camera is fixed (no pan/tilt/zoom during recording).

- -Court corners are visible in the first frame.

- -Players are the largest moving objects on court.

II. Literature Review & Theoretical Background

1. Perpective Transformation (Homography):

In computer vision, Homography is a projective transformation that maps points from one plane to another. For this project, we map the camera image plane to the real-world court plane.

Mathematical Formulation:
	The transformation is defined by a 3×3 matrix H:

X'Y'W'=h11h12h13h21h22h23h31h32h33×xy1

Where:

(x, y): pixel coordinates of camera image

(X, Y): real world coordinates

2. Motion detection via background subtraction

Since deep learning models are not allowed, we use Background Subtraction to identify moving objects:

-Algorithm: MOG2 (Mixture of Gaussians) from OpenCV (cv.createBackgroundSubtractorMOG2).

-Principle: Each pixel is modeled as a mixture of Gaussian distributions. Pixels that deviate significantly from the background model are classified as "foreground" (moving objects).

Parameters:

-history = 500: Number of frames used to build the background model.

-varThreshold = 50: Sensitivity threshold for foreground detection.

3. Contour detection and player filtering:

After obtaining the foreground mask:

-Morphological operations remove noise (opening, closing, dilation).

-Contour detection (cv.findContours) identifies connected regions.

Filtering by:

-Area (>800 pixels): Removes small objects (shuttlecock, noise).

-Aspect ratio (height>width): Filters out non-human shapes.

-Minimum height (30 pixels): Removes small false positives.

4. Foot position estimations

The foot position is crucial because it represents the player's actual contact point with the court surface. Given a bounding box (x,y,w,h):

-Foot X = x+w/2 (center of the bounding box width).

-Foot Y = y+h (bottom of the bounding box).

5. Heatmap generation

A heatmap visualizes spatial density:

-Accumulation: Each transformed foot position increments a 2D accumulator array.

-Smoothing: Gaussian blur (σ=20, kernel size 81×81) creates continuous density distribution.

-Normalization: Values are scaled to [0, 255] for visualization.

-Colormap: OpenCV's COLORMAP_JET maps density values to colors (blue = low, red = high).

III. System workflow

IV. Detail Implementations:

1. Calibration Module (in setup_perspective.py file)

Purpose: Compute the transformation matrix MM that maps camera pixels to real-world court coordinates.

Process:

-Load the first frame of the video.

-Display the frame at a manageable resolution (960px width).

-User clicks the 4 corners of the court in exact order:

+Corner 1: Top-Left (far left, near net)

+Corner 2: Top-Right (far right, near net)

+Corner 3: Bottom-Right (near camera, right side)

+Corner 4: Bottom-Left (near camera, left side)

Each click draws a marker and label on the image.

After collecting 4 points, compute:

src = np.float32(points)

dst = np.float32([

[0,        0       ],

[COURT_W,  0       ],

[COURT_W,  COURT_H ],

[0,        COURT_H ]

])

M = cv.getPerspectiveTransform(src, dst)

np.save(OUTPUT_M, M)

2. Player detection module:

def detect_players(fg_mask, min_area=MIN_AREA):

# Lọc nhiễu morphology

kernel = cv.getStructuringElement(cv.MORPH_ELLIPSE, (7, 7))

mask = cv.morphologyEx(fg_mask, cv.MORPH_OPEN,  kernel)

mask = cv.morphologyEx(mask,    cv.MORPH_CLOSE, kernel)

mask = cv.dilate(mask, kernel, iterations=2)

contours, _ = cv.findContours(mask, cv.RETR_EXTERNAL,

cv.CHAIN_APPROX_SIMPLE)

boxes = []

for cnt in contours:

area = cv.contourArea(cnt)

if area < min_area:

continue

x, y, w, h = cv.boundingRect(cnt)

if h < w:          # người đứng thì h > w

continue

if h < 30:         # quá thấp → không phải người

continue

boxes.append((x, y, w, h))

return boxes

3. Coordinates transformation module:

def transform_point(M, fx, fy):

pt = np.float32([[[fx, fy]]])

out = cv.perspectiveTransform(pt, M)

rx = int(out[0][0][0])

ry = int(out[0][0][1])

return rx, ry

4.Heatmap Accumulation

heatmap_acc = np.zeros((COURT_H, COURT_W), dtype=np.float32)

frame_count    = 0

positions_log  = []

print('[START] Bat dau xu ly video... Nhan Q de dung som.')

while True:

ret, frame = cap.read()

if not ret:

print('[INFO] Het video hoac khong doc duoc frame.')

break

if frame_count >= max_frames:

print(f'[INFO] Du {DURATION_MIN} phut. Dung xu ly.')

break

fg_mask = bg_sub.apply(frame)

boxes   = detect_players(fg_mask)

display = frame.copy()

frame_h = frame.shape[0]

split_y = int(frame_h * BOTTOM_HALF_RATIO)

for bbox in boxes:

x, y, w, h = bbox

center_y = y + h // 2

if center_y < split_y:

continue

frame_w  = frame.shape[1]

center_x = x + w // 2

if center_x < int(frame_w * CENTER_X_MIN) or center_x > int(frame_w * CENTER_X_MAX):

continue

cv.rectangle(display, (x,y), (x+w, y+h), COLOR_BOX, 2)

fx, fy = get_foot_point((x, y, w, h))

cv.circle(display, (fx, fy), 5, COLOR_FOOT, -1)

rx, ry = transform_point(M, fx, fy)

if 0 <= rx < COURT_W and 0 <= ry < COURT_H:

heatmap_acc[ry, rx] += 1

5. Visualization

HALF_Y = COURT_H // 2

acc_half = acc[HALF_Y:, :]  #bottom half of frame extract

blur = cv.GaussianBlur(acc_half, (81, 81), 20) #larger kernal like 81x81 ensure smooth

blur = np.power(blur, 0.4) #power transformation

norm = cv.normalize(blur, None, 0, 255, cv.NORM_MINMAX)

norm_uint8 = np.uint8(norm)

colored = cv.applyColorMap(norm_uint8, cv.COLORMAP_JET)

V. Implementation details

1. Key parameters

| Parameter | Value | Description |
| --- | --- | --- |
| COURT_W | 610 | In pixels (milimeters) |
| COURT_H | 1340 | In pixels |
| DURATION_MIN | 10 | Minutes |
| MIN_AREA | 800 | Minimum player contour area |
| BOTTOM_HALF_RATIO | 0.5 | Only 50% of bottom frame is processing |
| CENTER_X_MIN | 0.2 | Ignore left 20% frame |
| CENTER_X_MAX | 0.8 | Ignore right 20% frame |

2. Perform consideration:

Processing Speed:

-MOG2 background subtraction: ~5-10 ms per frame.

-Contour detection: ~2-5 ms per frame.

-Heatmap update: negligible.

-Total: ~10-15 ms/frame → ~60-100 FPS processing capability.

Memory Usage:

-Heatmap accumulator: 1340 × 610 × 4 bytes = ~3.3 MB.

-Frame buffers: ~2-3 MB per frame.

VI. Code Walkthrough (main.py program)

1. Main processing loop

def main():

try:

M = np.load(M_PATH)

print(f'[OK] Da load ma tran M tu {M_PATH}')

except FileNotFoundError:

print(f'[LOI] Khong tim thay {M_PATH}')

print('     Hay chay perspective_setup.py truoc!')

return

cap = cv.VideoCapture(VIDEO_PATH)

if not cap.isOpened():

print(f'[LOI] Khong mo duoc video: {VIDEO_PATH}')

return

fps = cap.get(cv.CAP_PROP_FPS)

if fps <= 0:

fps = 30

max_frames = int(fps * 60 * DURATION_MIN)

print(f'[INFO] FPS={fps:.1f} | Se xu ly {max_frames} frames ({DURATION_MIN} phut)')

bg_sub      = cv.createBackgroundSubtractorMOG2(

history=500, varThreshold=50, detectShadows=True)

heatmap_acc = np.zeros((COURT_H, COURT_W), dtype=np.float32)

frame_count    = 0

positions_log  = []

print('[START] Bat dau xu ly video... Nhan Q de dung som.')

while True:

ret, frame = cap.read()

if not ret:

print('[INFO] Het video hoac khong doc duoc frame.')

break

if frame_count >= max_frames:

print(f'[INFO] Du {DURATION_MIN} phut. Dung xu ly.')

break

fg_mask = bg_sub.apply(frame)

boxes   = detect_players(fg_mask)

display = frame.copy()

frame_h = frame.shape[0]

split_y = int(frame_h * BOTTOM_HALF_RATIO)

for bbox in boxes:

x, y, w, h = bbox

center_y = y + h // 2

if center_y < split_y:

continue

frame_w  = frame.shape[1]

center_x = x + w // 2

if center_x < int(frame_w * CENTER_X_MIN) or center_x > int(frame_w * CENTER_X_MAX):

continue

cv.rectangle(display, (x,y), (x+w, y+h), COLOR_BOX, 2)

fx, fy = get_foot_point((x, y, w, h))

cv.circle(display, (fx, fy), 5, COLOR_FOOT, -1)

rx, ry = transform_point(M, fx, fy)

if 0 <= rx < COURT_W and 0 <= ry < COURT_H:

heatmap_acc[ry, rx] += 1

positions_log.append((rx, ry, frame_count))

elapsed_min = frame_count / (fps * 60)

cv.putText(display,

f'Frame: {frame_count}/{max_frames} | '

f'Time: {elapsed_min:.1f}/{DURATION_MIN} min | '

f'Players: {len(boxes)}',

(10, 30), cv.FONT_HERSHEY_SIMPLEX, 0.65, (150,255,0), 2)

h_disp, w_disp = display.shape[:2]

scale = DISPLAY_W / w_disp

display_small = cv.resize(display, (DISPLAY_W, int(h_disp * scale)))

cv.imshow('Badminton Tracking', display_small)

if cv.waitKey(1) & 0xFF == ord('q'):

print('[INFO] Nguoi dung dung som.')

break

frame_count += 1

cap.release()

cv.destroyAllWindows()

print(f'\n[INFO] Tong so diem vi tri da thu thap: {len(positions_log)}')

if len(positions_log) == 0:

print('[CANH BAO] Khong co du lieu vi tri. Kiem tra lai video va M.npy')

return

result = render_heatmap(heatmap_acc, COURT_IMG)

cv.imwrite(OUTPUT_PATH, result)

print(f'[OK] Da luu heatmap vao: {OUTPUT_PATH}')

h_r, w_r = result.shape[:2]

scale_r   = HEATMAP_DISP_W / w_r

result_small = cv.resize(result, (HEATMAP_DISP_W, int(h_r * scale_r)))

cv.imshow('Heatmap Ket qua', result_small)

print('Nhan phim bat ky de thoat...')

cv.waitKey(0)

cv.destroyAllWindows()

if __name__ == '__main__':

main()

2. Player detection function (bottom half player on camera side)

def detect_players(fg_mask, min_area=MIN_AREA):

# Lọc nhiễu morphology

kernel = cv.getStructuringElement(cv.MORPH_ELLIPSE, (7, 7))

mask = cv.morphologyEx(fg_mask, cv.MORPH_OPEN,  kernel)

mask = cv.morphologyEx(mask,    cv.MORPH_CLOSE, kernel)

mask = cv.dilate(mask, kernel, iterations=2)

contours, _ = cv.findContours(mask, cv.RETR_EXTERNAL,

cv.CHAIN_APPROX_SIMPLE)

boxes = []

for cnt in contours:

area = cv.contourArea(cnt)

if area < min_area:

continue

x, y, w, h = cv.boundingRect(cnt)

if h < w:          # người đứng thì h > w

continue

if h < 30:         # quá thấp → không phải người

continue

boxes.append((x, y, w, h))

return boxes

3. Heatmap rendering function

def render_heatmap(acc, court_img):

HALF_Y = COURT_H // 2

acc_half = acc[HALF_Y:, :]

blur = cv.GaussianBlur(acc_half, (81, 81), 20)

if blur.max() == 0:

print('[WARN] Chua co du lieu heatmap!')

court = np.ones((HALF_Y, COURT_W, 3), dtype=np.uint8)

court[:] = (60, 140, 60)

_draw_half_court_lines(court)

return court

blur = np.power(blur, 0.4)

norm = cv.normalize(blur, None, 0, 255, cv.NORM_MINMAX)

norm_uint8 = np.uint8(norm)

colored = cv.applyColorMap(norm_uint8, cv.COLORMAP_JET)

court = np.ones((HALF_Y, COURT_W, 3), dtype=np.uint8)

court[:] = (60, 140, 60)

result = cv.addWeighted(court, 0.3, colored, 0.7, 0)

_draw_half_court_lines(result)

_draw_legend(result)

return result

4. Smoothtracker filtering program (for the large boundary only).

class SmoothTracker:

def __init__(self):

self.alpha     = SMOOTH_ALPHA

self.miss_tol  = MISS_TOLERANCE

self._smooth   = None   # (cx, cy, w, h) float

self._miss     = 0

self._last_vel = (0.0, 0.0)   # vận tốc cx, cy để dự đoán khi mất

def _clamp(self, w, h):

w = float(np.clip(w, MIN_BOX_W, MAX_BOX_W))

h = float(np.clip(h, MIN_BOX_H, MAX_BOX_H))

return w, h

def update(self, roi_boxes):

"""

roi_boxes : list (x,y,w,h) đã lọc ROI.

Chỉ lấy box lớn nhất (người gần camera).

Trả về (x,y,w,h) đã smooth hoặc None nếu mất quá lâu.

"""

if roi_boxes:

# Lấy box có diện tích lớn nhất

best = max(roi_boxes, key=lambda b: b[2] * b[3])

bx, by, bw, bh = best

bw, bh = self._clamp(bw, bh)

cx = float(bx + bw / 2)

cy = float(by + bh / 2)

if self._smooth is None:

self._smooth = (cx, cy, bw, bh)

self._last_vel = (0.0, 0.0)

else:

a = self.alpha

scx, scy, sw, sh = self._smooth

new_cx = a * cx  + (1-a) * scx

new_cy = a * cy  + (1-a) * scy

new_w  = a * bw  + (1-a) * sw

new_h  = a * bh  + (1-a) * sh

self._last_vel = (new_cx - scx, new_cy - scy)

self._smooth = (new_cx, new_cy, new_w, new_h)

self._miss = 0

else:

self._miss += 1

if self._miss > self.miss_tol:

self._smooth = None

self._last_vel = (0.0, 0.0)

elif self._smooth is not None:

# Dự đoán vị trí bằng vận tốc cuối (coast)

scx, scy, sw, sh = self._smooth

vx, vy = self._last_vel

# Giảm dần vận tốc khi không có detection

vx *= 0.85

vy *= 0.85

self._last_vel = (vx, vy)

self._smooth = (scx + vx, scy + vy, sw, sh)

if self._smooth is None:

return None

cx, cy, w, h = self._smooth

x = int(cx - w / 2)

y = int(cy - h / 2)

return (x, y, int(w), int(h))

VII. Result:

1. Sample tracking visualization:

Boundary box: the green rectangular box around detected player.

Foot prints: red dots marking the foot positions

Status bar: turquoise color bar on the top left of screen showing frame count, running time and player detected.

2. Heatmap result:

Heatmap after 10 minutes of running

Output file: heatmap_result.png

Court diagram: Green background with white court lines

Heatmap overlay: JET colormap showing density

Legend (on the bottom right): color bar indicate activity level (red = highest, blue = lowest)

Interpretation:

-Red zones: Player frequently moved to these areas (e.g., baseline corners, net area).

-Blue zones: Player rarely moved to these areas (e.g., sidelines, back corners).

-Other colors zones: Moderate activity areas.